NBS:

Project:

Date:

H43 METAL COMPOSITE PANEL CLADDING / COVERING

To be read with preliminaries / general conditions.

TYPE OF CLADDING SYSTEM

155 ROOFLIGHTS TO METAL ROOF CLADDING

- Drawing reference(s): _________

- Supports: Kingspan Multibeam cold rolled purlins, maximum 3000mm centres, subject to project wind and snow loadings.

- Bearing width - end lap: Minimum 60mm, intermediate: 50mm (for in-plane Factory Assembled Unit (FAU)).

- Pitch: Minimum 0.72° after deflection.

- Manufacturer and reference: Kingspan Insulated Panels, polycarbonate rooflight / roof cladding system for standard internal and external environments.

- Safety: Both barrel vault and FAU classified non-fragile Class B in accordance with ACR [M]001: 2014.

- Profile reference: Kingspan Day-Lite Vault, KS1000 DLVLT.

- Material (FAU):

- 1mm thick profiled polycarbonate liner (internal face of FAU) Kingspan Day-Lite products fully satisfy the requirements of the National Building Regulations achieving Euro Class B internally and National Class AA and Euro Class BROOF(t4) externally. Kingspan Day-Lite Vault achieves a classification of B-s1,d0 when tested to EN 13501-1: 2007.

- High density polyethylene foam spacer units between internal and outer facings of FAU at purlin locations.

- 16mm thick multiwall polycarbonate sheet (external face of FAU).

- Colour: Kingspan Day-Lite (Clear / Opal only).

- Material (barrel vault):

- 2mm thick profiled polycarbonate nominally 1245mm wide.

- Colour: Kingspan Day-Lite (Clear / Opal White only).

- End laps (above rooflight):

- FAU: 60mm single run of 6mm x 5mm butyl rubber sealant internally.

- FAU: film-backed butyl tape applied over joint (100mm wide) externally.

- End laps (FAU to FAU):

- FAU: 60mm single run of 6mm x 5mm butyl rubber sealant internally.

- FAU: Film-backed butyl tape applied over joint (100mm wide) externally.

- FAU: 25mm double-sided tape applied to end lap externally.

- End laps (below rooflight):

- FAU: 60mm single run of 6mm x 5mm butyl rubber sealant internally.

- FAU: Film-backed butyl tape applied over joint (100mm wide) externally.

- FAU: 25mm double-sided tape applied to end lap externally.

- End laps (barrel vault to barrel vault):

- 2 number runs of polycarbonate compatible sealant, located on the crown and leading valley edge.

- Single run of polycarbonate compatible sealant applied on side lap for full length of end lap.

- Side laps:

- FAU: single run of factory-applied foam tape.

- FAU: single run of 50mm wide film-backed butyl tape applied to external lap.

- Side laps (kerb to panel):

- Un-reinforced membrane typically 1.2mm thick welded to panel and kerb, minimum 50mm lap on to panel.

- Side laps (barrel vault to kerb):

- Single run of 6mm x 5mm butyl rubber sealant around perimeter.

- Fasteners: As determined by clause 220A.

- Number and location (FAU):

- End laps (above rooflight FAU): Five no. at panel ends, as recommended by cladding manufacturer.

- End laps (rooflight to rooflight FAU): Five no. fix through every profile in the liner sheet.

- End laps (below rooflight FAU): Five no. fix through every profile in the liner sheet.

- Number and location (kerb to panel at top and bottom / kerb to panel at jamb):

- 1 through fixing located on outer edge of kerb to panel liner at 450mm centres maximum, subject to design.

- Quantity of through fixings per kerb, located at purlins, to suit wind-loading.

- Quantity of through fixings per kerb, to panel liner at jamb kerb ends, subject to wind-loading.

- Number and location (barrel vault to kerb):

- Top, bottom and jamb: Fixing located at 333mm centres maximum to kerb upstand.

- Barrel Vault end lap: 1 number located through both barrel vaults, fixings located above crown and 1 number through lower barrel vault, located below crown.

- Accessories:

- Kerb upstand: Z45 galvanised steel typically 1.8mm thick subject to design.

- Thermal transmittance (U-value) of 1.1W/m²K calculated using the method required by the Building Regulations Part L2 (England and Wales) and Building Standards Section 6 (Scotland).

- Manufacturers 25 year product guarantee required.

- Air leakage rate of 3m³/hr/m² at 50Pa, based on the assumption that the full building envelope is constructed using Kingspan panels.

- Other requirements:

- Panels manufactured under the following ISO standards: ISO 9001 (Quality), ISO 14001 (Environmental) and OHSAS 18001 (Health & Safety).

- Cladding contractor to attend a product training course at Kingspan Insulated Panels.

# 197A ATTACHMENT

- Determine the number and location of cladding fasteners recommended by the cladding manufacturer to resist wind loads calculated in accordance with BS 6399: Part 2 Standard Method and BS 5427: Part 1.

- Calculate wind loads on roof and wall cladding appropriate to location, exposure, roof height, building shape and size in accordance with BS 6399: Part 2 Standard Method and BS 5427: Part 1.

- Basic wind speed (Vb): ___ m/s.

- Altitude factor (Sa): ___

- Direction factor (Sd): ___

- Seasonal factor (Ss): 1

- Probability factor (Sp): 1

- Terrain and building factor (Sb): ___

- External and internal size effect factors (Ca): 1.

- External pressure coefficients (Cpe): As determined from BS 6399: Part 2, clauses 2.4 and 2.5.

- Internal pressure coefficients (Cpi): As determined from BS 6399: Part 2, clause 2.6.

- Dominant opening: _________

# 220A FASTENER

- Primary fastener - self coring (FAU to panel end lap):

- Type(s), size(s) and drilling capacity: As recommended by fastener manufacturer to suit type and thickness of supports, and thickness of cladding panels.

- Screw material: Anti-corrosion coated carbon steel.

- Secondary fasteners: Self coring fasteners.

- Screw / washer material: As primary fasteners.

- Primary fasteners - High threaded screws with bonded washers (FAU end lap & intermediate purlins):

- Type(s), size(s) and drilling capacity: As recommended by fastener manufacturer to suit type and thickness of supports, and thickness of cladding panels.

- Screw material: Grade 304 Austenitic Stainless Steel.

- Washer material: Non-ferrous.

- Washer size: 29mm diameter.

- Heads: PPC head or coloured plastic heads.

- Primary fasteners - Self drilling self-tapping fastener (kerb to purlin and to panel liner):

- Type(s), size(s) and drilling capacity: As recommended by fastener manufacturer to suit type and thickness of supports, and thickness of cladding panels.

- Screw material: Grade 304 Austenitic Stainless Steel.

- Washer material: Non-ferrous.

- Heads: Irius head.

- Primary fasteners – Self-drilling, self-tapping fastener (kerb to panel liner, also kerb to panel at intermediate locations subject to design):

- Type(s), size(s) and drilling capacity: As recommended by fastener manufacturer to suit type and thickness of supports, and thickness of cladding panels.

- Screw material: Grade 304 Austenitic Stainless Steel.

- Washer material: Non-ferrous.

- Heads: Low profile.

- Primary fasteners - Self-drilling, self-tapping fastener (barrel vault to kerb):

- Type(s), size(s) and drilling capacity: As recommended by fastener manufacturer to suit type and thickness of supports, and thickness of cladding panels.

- Screw material: Grade 304 Austenitic Stainless Steel.

- Washer material: Non-ferrous.

- Washer size: 29mm diameter.

- Heads: PPC head or coloured plastic heads.